Propidium Monoazide (PMA) Treatment Protocol Development

Column 1 of the following table gives the 7 basic steps on the PMA method: 1) Pre-processing, 2) Transfer of samples to containers, 3) Addition of PMA to treatment samples, 4) Addition of solution to controls, 5) Incubation in the dark, 6) Incubation under a halogen lamp, 7) DNA extraction. Columns 2-5 of the table outline the PMA procedures used by four recent studies (Ramírez et al., 2018; Amiraux et al., 2019; Burkert et al., 2019; Malard et al., 2019). Columns 6-9 of the table outline 2 prospective protocols for ice and snow samples (pre-filtering and no pre-filtering), followed by 2 prospective protocols for cryoconite (pre-filtering and no-pre-filtering).

| Step | Antarctic Snow (Malard et al., 2019) | Alaskan Permafrost (Burkert et al., 2019) | Marine Sediment (Ramírez et al., 2018) | Sea Ice (Amiraux et al., 2019) | Ice/Snow Protocol 1 (Pre-filtering) | Ice/Snow Protocol 2 (No pre-filtering) | Cryoconite Protocol 1 (Pre-filtering) | Cryoconite Protocol 2 (No pre-filtering) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. Pre-processing | Melted samples passed through ﻿0.2 µm Sterivex filters. Frozen at -20C for transport to UK. | Cells separated from soil matrix by disrupting 5 g of sample in 25 mL of 1% sodium hexametaohosphate buffer (SHMP) and sonicating for 1 min at 20 V using ﻿QSonica ultrasoni- cator with a 0.6-cm probe. Leave samples for 15 mins to settle. Transfer supernatant to clean 50 mL tube. Add ﻿15 mL of 1% SHMP to the pellet and sonicated it again with a 0.6-cm probe for 1 min at 20 V. Leave samples for 15 minutes to settle. ﻿Combine new supernatant with supernatant from the previous step. ﻿Centrifuged the combined supernatant at 20 X g for 2 min. Transfer supernatant to 1 x 50 mL tube for PMA treatment and 1 x 50 mL tube for control. ﻿Centrifuge at 10,000 X g for 15 min to pellet the biomass. PMA group is ready for immediate use. | Use sterile cut syringes to aseptically sample sediment cores collected from Arctic and Pacific sites. | Sea ice samples concentrated by pelleting at 5,000 x g for 5 mins. Pellets then homogenized in 1 mL of 0.2 µm filtered sea water. |  |  |  |  |
| 2. Transfer samples to containers | Cut filters in half using sterile ﻿and DNAase/ethanol treated razors. | ﻿Resuspended extracted PMA-treatment cell pellets in 500 µl of 0.85% NaCl solution and placed in clear 1.5 mL microcentrifuge tubes. | Place 0.1 cm3 of fresh wet sediment in six 1.5 mL translucent microcentrifuge tubes. |  |  |  |  |  |
| 3. Add PMA to treatment samples | To PMA treatment samples, add 20 mM stock solution of PMA in a 20% (v/v) aqueous solution of ﻿dimethyl sulfoxide (DMSO), to a final concentration of ﻿100 µM. | ﻿To PMA treatment samples, add 2.5 µl of 20 mM propidium monoazide solution to each microcentrifuge tube, to a final concentration of 100 µM. | To 3 of the tubes (our PMA treatment samples), add PMA to a final concentration of ﻿20 µM in 1× sterile phosphate buffer saline in a final volume of 1 ml. | Add PMA to half of samples designated PMA-treatment samples, at a final concentration of 100 µM. |  |  |  |  |
| 4. Add solution to non-PMA treated samples | To non-PMA treated samples add ﻿a 20% (v/v) aqueous solution of DMSO. | Control group pellets to be stored at -20C ready for DNA extraction. There is no further PMA-control as this study referenced a different experiment to estimate DNA removal efficiency of PMA. | To the 3 remaining, untreated controls, add ﻿1× phosphate buffer saline to 1 ml volume. | For the half of samples designated for no treatment, store at -80C. |  |  |  |  |
| 5. Incubate in the dark | Incubate treated and untreated well plates on ice for 10 minutes in the dark with occasional mixing. | Incubate PMA tubes in the dark at room temperature for 10 mins. | Incubate all samples in darkness at 25C for 1 hour, with slow mixing via hand inversion every 5 mins. | Incubate in the dark at 4C for 5 mins while mixing occasionally. |  |  |  |  |
| 6. Incubate under a lamp | Incubate treated and untreated well plates 20cm from a 650 W halogen lamp for 5 minutes. | Place PMA tubes on a sheet of foil in an ice bucket to prevent warming. Place a 500 W halogen lamp 20cm above the samples for 15 mins. Mix samples gently every 5 mins. After, centrifuge samples at 10,000 X g for 15 mins and discard supernatant. | Place all samples on ice and expose to 500 W halogen lamp for 15 minutes at 15 cm distance, whilst hand mixing via inversion every 5 minutes. | Filter samples through 0.8 µm Whatman nucleopore polycarbonate filters (24 mm, autoclaved 1 h at 110C prior to use) to retain attached bacteria on the filters. Place filters in petri dish on ice and expose to light for 2 mins using 650 W halogen lamp, at 20 cm distance. |  |  |  |  |
| 7. Extract DNA | Wash all samples twice with phosphate buffer saline (PMS) prior to DNA extraction. | Store PMA pellets at -20C until DNA extraction. | Store samples at -80C until DNA extraction. | Extract DNA from treated and untreated filters. |  |  |  |  |

References

Amiraux, R., Burot, C., Bonin, P., Guasco, S., Babin, M., and Rontani, J.-F. (2019). Stress Factors Resulting From The Arctic Vernal Sea Ice Melt: Impact On The Viability Of The Bacterial Communities Associated To Sympagic Algae. Elem Sci Anth Submitted.

Burkert, A., Douglas, T. A., Waldrop, M. P., and Mackelprang, R. (2019). Changes in the active, dead, and dormant microbial community structure across a pleistocene permafrost chronosequence. Appl. Environ. Microbiol. 85. doi:10.1128/AEM.02646-18.

Malard, L. A., Šabacká, M., Magiopoulos, I., Mowlem, M., Hodson, A., Tranter, M., et al. (2019). Spatial variability of Antarctic surface snow bacterial communities. Front. Microbiol. 10, 1–12. doi:10.3389/fmicb.2019.00461.

Ramírez, G. A., Jørgensen, S. L., Zhao, R., and D’Hondt, S. (2018). Minimal Influence of Extracellular DNA on Molecular Surveys of Marine Sedimentary Communities. Front. Microbiol. 9, 1–12. doi:10.3389/fmicb.2018.02969.