Methods for measuring dissolved organic carbon

The following methods are taken directly from the text of the referenced paper, with minor changes.

Summary of methods:

| Initial collection | Melting | Filtering | Acidification | Analysis | Study |
| --- | --- | --- | --- | --- | --- |
| Sterile WhirlPak plastic bags | Melted and water collected in combusted glass or Teflon bottles | 0.2m Sterivex cartridges, pre-cleaned by soaking in 10% HCl + rinsing with 20L Milli-Q water or a combusted GFF (Whatman) pre-filter and a combusted 0.2 m Anodisc membrane | Acidify and store in combusted vial. | Quantified as non-pugeable organic carbon (NPOC) by high temperature combustion with a Shimadzu TOC-VCSH analyser + high sensitivity platinum catalyst. | (Bhatia et al., 2010) |
| 250mL glass bottles combusted at 450C for 4 h. Rinsed 3 times with sample filtrate prior to collection | NA – used stream samples | Combusted glass filtration apparatus, through a combusted pre-weighed glass-fiber pre-filter ﻿(nominal pore-size 0.7-m) and a combusted 0.2-m Anodisc membrane. | Acidified to pH 2 with concentrated HCl, and stored in a 40-mL glass vial. | Quantified as non-pugeable organic carbon (NPOC) by high temperature combustion with a Shimadzu TOC-VCSH analyser + high sensitivity platinum catalyst. | (Bhatia et al., 2013) |
| Collected using precleaned polyethylebe shovel. Stored in acid-washed high-density polyethylene bottles (wide-mouth 4L). | Melted at room temp | Precleaned Whatman Polycap 36TC filters (0.2-m). |  | High-temperature nonpurgeable organic carbon analysis on a Shimadza TOC-VCSH analyser using a high-sensitivity catalyst to enable detection of low DOC concentrations | (Fellman et al., 2015) |
| Sterile Whirl-Pak bags. | Melted at ambient temperature (10C) | Pre-cleaned and pre-furnaced glass filtration apparatus (0.70 m GF/F filters) into pre-furnaced borosilicate amber glass bottles. | Acidified to pH 2–3 with concentrated HCl and stored at ≤4 °C | Non-purgeable organic carbon by high-temperature combustion (680 °C), using a Shimadzu TOC-VCSN/TNM-1 Analyser equipped with a high sensitivity catalyst. | (Musilova et al., 2017) |

(Bhatia et al., 2010) Molecular characterization of dissolved organic matter associated with the Greenland ice sheet

- Snow samples collected aseptically using sterile WhirlPak plastic bags and melted onsite in warm waterbath – conditions in the field precluded melting the samples at controlled 4C.

- Water samples collected in either combusted glass or acid-cleaned Teflon bottles.

- Samples filtered onsite through 0.2 m filters prior to extraction.

- Most samples filtered using 0.2m Sterivex cartridges, that had been pre-cleaned by soaking in 10% HCl bath for at least one day, followed by rinsing with 20L Milli-Q water.

Converting M to mg L-1

1M = 35,500 mg L-1

1M = 1,000,000 M

1M = 0.0355 mg L-1

9M = 319,500 mg L-1

9M = 0.3195 mg L-1

Background DOC concentrations of the pre-cleaned units was approximately 9M (0.32 mg L-1)

- Due to limited availability of pre-cleaned Sterivex units in the field, the remaining samples were filtered through a combusted GFF (Whatman) pre-filter and a combusted 0.2 m Anodisc membrane.

- The final volumes of 0.2 m filtrate differed to accommodate a range of anticipated DOC contents as well as the difficulties encountered when filtering some samples (volume filtered 87mL – 15L).

- An aliquot of the 0.2 m filtrate was acidified and stored in a combusted vial for DOC analysis.

- Total and dissolved organic carbon (TOC, DOC) concentrations were quantified as non-pugeable organic carbon (NPOC) by high temperature combustion (680C) with a Shimadzu TOC-VCSH analyser equipped with a high sensitivity platinum catalyst.

- Samples were quantified using a 5-point standard curve made with potassium hydrogen phthalate (KHP).

- Blanks and reference standards were analysed routinely within each sample run.

- ﻿Reference standards for low car- bon water and deep-sea water were obtained from the Consensus Reference Materials Project, Hansell Laboratory, University of Miami.

(Bhatia et al., 2013) Organic carbon export from the Greenland ice sheet

- All glasswear was combusted at 450C for at least 4 h and all Teflon- and plastic-ware was soaked overnight in 10% hydrochlorice (HCl) acid and rinsed extensively with Milli-Q water.

- Samples were collected in bottles that were rinsed three times with sample (or filtrate, as appropriate) prior to collection.

- Samples for DOC analyses were collected in 250mL glass bottles. All samples were filtered on-site using a combusted glass filtration apparatus, through a combusted pre-weighed glass-fiber pre-filter ﻿(GF/F; Whatman; nominal pore-size 0.7-lm) and a combusted 0.2-lm Anodisc mem- brane (Whatman).

- ﻿The GFF pre-filter was kept as cold as possible in the field, and frozen at -20 °C upon return to the laboratory for POC analyses.

- The 0.2-m filtrate was acidified to pH 2 with concentrated HCl, and stored in a 40-mL glass vial. Samples were kept as cold as possible in the field and stored at 4 °C upon return to the laboratory.

- DOC concentrations were quantified as described in Bhatia et al. (2010). The limit of detection (based on instrument blanks) was 0.02 mg L-1. Analytical error was less than 2% (±0.01 mg L-1) based on replicate injections.

(Fellman et al., 2015) Spatial Variation in the Origin of Dissolved Organic Carbon in Snow on the Juneau Icefield, Southeast Alaska

- Snow samples were collected in the accumulation zone of the Juneau Icefield on a single day in May.

- Snow samples were collected using a precleaned polyethylebe shovel and stored in acid-washed high-density polyethylene bottles (wide-mouth 4L).

- Samples were allowed to melt at room temperature and immediately filtered through precleaned Whatman Polycap 36TC filters (0.2-m).

- Samples for DOC concentration were analysed immediately following filtration.

- Concentrations of DOC were determined by a high-temperature nonpurgeable organic carbon analysis on a Shimadza TOC-VCSH analyser using a high-sensitivity catalyst to enable the detection of low DOC concentrations (see Stubbins and Dittmar, 2012)

- Analytical precision for the DOC was no more than 0.02 mg C L-1, which was determined from the standard deviation of identical samples reanalysed during the sample run.

(Musilova et al., 2017) Microbially driven export of labile organic carbon from the Greenland ice sheet

- Samples were collected aseptically into sterile Whirl-Pak bags.

- Snow and ice samples were melted at ambient temperature (10C) upon transportation to the field camp laboratory.

- Samples were filtered immediately through a pre-cleaned and pre-furnaced glass filtration apparatus into pre-furnaced borosilicate amber glass bottles.

- Pre-furnaced 0.70 m GF/F (Millipore) filters were used for DOC analyses.

- Filtrates for DOC analyses were acidified to pH 2–3 with concentrated HCl and stored at ≤4 °C in the field laboratory and during transport and storage at the University of Bristol

- Triplicate procedural blanks were carried out by collecting autoclaved Milli-Q water into the same containers/Whirl-Pak bags as the samples, filtering it and storing it using the same procedure as applied for the samples.

- DOC concentrations were measured in all sample types as non-purgeable organic carbon by high-temperature combustion (680 °C), using a Shimadzu TOC-VCSN/TNM-1 Analyser equipped with a high sensitivity catalyst, following the methods described by Lawson et al., 2014.

References

Bhatia, M. P., Das, S. B., Longnecker, K., Charette, M. A., and Kujawinski, E. B. (2010). Molecular characterization of dissolved organic matter associated with the Greenland ice sheet. Geochim. Cosmochim. Acta 74, 3768–3784. doi:10.1016/j.gca.2010.03.035.

Bhatia, M. P., Das, S. B., Xu, L., Charette, M. A., Wadham, J. L., and Kujawinski, E. B. (2013). Organic carbon export from the Greenland ice sheet. Geochim. Cosmochim. Acta 109, 329–344. doi:10.1016/j.gca.2013.02.006.

Fellman, J. B., Hood, E., Raymond, P. A., Stubbins, A., and Spencer, R. G. M. (2015). Spatial Variation in the Origin of Dissolved Organic Carbon in Snow on the Juneau Icefield, Southeast Alaska. Environ. Sci. Technol. 49, 11492–11499. doi:10.1021/acs.est.5b02685.

Lawson, E. C., Wadham, J. L., Tranter, M., Stibal, M., Lis, G. P., Butler, C. E. H., et al. (2014). Greenland ice sheet exports labile organic carbon to the arctic oceans. Biogeosciences 11, 4015–4028. doi:10.5194/bg-11-4015-2014.

Musilova, M., Tranter, M., Wadham, J., Telling, J., Tedstone, A., and Anesio, A. M. (2017). Microbially driven export of labile organic carbon from the Greenland ice sheet. Nat. Geosci. 10, 360–365. doi:10.1038/ngeo2920.

Stubbins, A., and Dittmar, T. (2012). Low volume quantification of dissolved organic carbon and dissolved nitrogen. Limnol. Oceanogr. Methods 10, 347–352. doi:10.4319/lom.2012.10.347.