Methods for measuring dissolved organic carbon

A) What levels of dissolved organic carbon are usually detected on glaciers?

Gao et al., (2020) Characterization, sources and transport of dissolved organic carbon and nitrogen from a glacier in the Central Asia

Muz Taw glacier in Central Asia - Average DOC concentrations in surface snow: 1.12 ± 1.66 mg L-1, in snow pit (100-110cm deep): samples 0.21 ± 0.04 mg L-1. Average DOC concentrations across glaciers/snow cover/ice sheets given as 0.06–3.40 mg L−1 with higher values in mountain glaciers.

Zhang et al., (2019) Dissolved organic carbon in snow cover of the Chinese Altai Mountains, Central Asia: Concentrations, sources and light-absorption properties

DOC in snow cover 1.01 ± 0.52 mg L−1 – but higher than Arctic glaciers.

B) How is dissolved organic carbon usually measures?

Gao et al., (2020) Characterization, sources and transport of dissolved organic carbon and nitrogen from a glacier in the Central Asia

- Collect snow and store in precleaned 60mL square-shaed polycarbonate bottles.

- Snow samples for major ions collected at the same time and stored in polythene bottles.

- Samples kept frozen until analysis.

Zhang et al., (2019) Dissolved organic carbon in snow cover of the Chinese Altai Mountains, Central Asia: Concentrations, sources and light-absorption properties

- Before sampling polycarbonate bottles pre-cleaned by washing three times with ultrapure water, soaking with 1MHCl for 24hrs, rinsing three times with ultrapure water, then soaking in ultrapure water for 24hrs.

- Snow directly collected into 30mL pre-cleaned bottles.

- Samples kept frozen and in dark until analysis.

- Samples filtered as soon as possible after melting in laboratory using a polytetrafluoroethylene (PTPE) membrane filter with 0.45 μm pore size.

- At least one blank made for every sampling process. Clean plastic gloves and dustproof clothes worn. Average DOC of blanks 58g L-1 < 6% average snow DOC.

- DOC concentrations were then determined using a Shimadzu TOC-5000A Total Organic Carbon analyser. LOD = 15g L-1.

Bhatia et al., (2010) Molecular characterization of dissolved organic matter associated with the Greenland ice sheet

Bhatia et al., (2013) Organic carbon export from the Greenland ice sheet

Fellman et al., (2013) Spatial Variation in the Origin of Dissolved Organic Carbon in Snow on the Juneau Icefield, Southeast Alaska

Musilova et al., (2017) Microbially driven export of labile organic carbon from the Greenland ice sheet

C) What can we infer about different levels of dissolved organic carbon on glaciers?