﻿Propidium Monoazide (PMA) treatment of samples

What is the question we’re asking?

What DNA comes from intact cells vs DNA from compromised cells (broken membrane) and exDNA (extracellular DNA).

How will we answer this question?

To achieve this, we need parallel samples:

- Samples treated with PMA (will give us just the DNA from intact cells)

- Control samples (will give us all DNA)

PMA binds to eDNA and DNA from cells with compromised membranes. PMA bound DNA cannot be amplified during the polymerase chain reaction (PCR).

PMA Method

- Add PMA to treatment samples

- Incubate for 1 hour while mixing

- Expose treatment samples to light (500W halogen bulb) for 1 hour while mixing

- Incubate control samples (without adding PMA) for 1 hour while mixing

- Expose control samples to light (500W halogen bulb) for 1 hour while mixing

Quantitative polymerase chain reaction (a.k.a. real-time PCR):

- Subtract PMA treatment from control to give absolute number of compromised cells

- Compare ASV frequency and abundance of PMA treated and control samples to determine the contribution of compromised cells and exDNA to diversity.

*IMPORTANT: Clean-up of DNA is important as residual PMA interferes with some qPCR reagents – would this be done using ﻿AMPure XP beads?

Papers

(Ramírez et al., 2018) Minimal Influence of Extracellular DNA on Molecular Surveys of Marine Sedimentary Communities

This paper looked at the impact of exDNA on sequencing surveys of marine sediments:

- Quantified DNA from intact cells vs exDNA or compromised cells was quantified

- Quantified ecological metrics and diversity indices for bulk DNA and live DNA

- exDNA and DNA from compromised cells was found to have minimal impact on diversity indices

ABSTRACT SUMMARY

- In near seafloor sediment exDNA is thought to contribute a large proportion to total DNA.

- If exDNA (from dead or moribund – near death - cells) DOES continue significantly to total DNA then this has serious implications for sequencing surveys of marine sediment.

- To address this knowledge gap ﻿Propidium Monoazide (PMA), a photoactive DNA-binding dye, was applied to sediment samples while surveying 16S rRNA genes.

- Only 3/12 shallow samples exhibited significant differences between relative abundance of total (intracellular + extracellular) Bacterial 16S rRNA genes and intracellular 16S rRNA genes.

- There was no significant differences between total and intracellular relative abundances of 16S rRNA genes.

- This investigation showed that even when there was a greater amount (abundance) of exDNA, it did not significantly effect community composition.

METHOD

PMA Treatments

- Sterile syringes used for aseptically sampling sediment mini-cores.

- For each sample depth horizon, add 0.1cm3 of sediment from mini-cores to each of six 1.5 ml translucent microcentrifuge tubes.

- Add PMA to three randomly chosen tubes at a concentration of 20 M in 1x sterile phosphate buffer saline in a final volume of 1 ml.

- The remaining 3 tubes are no-PMA controls. Add 1x 1ml phosphate buffer saline.

- Incubate all PMA-treated samples and no-PMA controls in darkness at 25C and 1 atmospheric pressure (atm) for 1 hour, while mixing slowly by hand via inversion every 5 minutes.

- Put all PMA-treated sample and non-PMA controls on ice and expose to a 500 W halogen lamp at a distance of 15 cm for 15 minutes, while hand mixing via inversion every 5 minutes.

- Store all samples at -80C and return to UK for DNA extraction, 16S rRNA gene quantification and sequencing.

PMA Treatments Rationale

- The efficacy of using PMA to distinguish between intracellular and exDNA relies on cell membrane integrity.

- As this study deals with extremely old sediment samples, intact membranes may be susceptible to damage.

- The protocol describes above focuses on minimizing artifactual exDNA by preventing cell damage within the process.

- Cell damage is minimized by 1) working with fresh (not frozen) samples, 2) hand mixing rather than vortexing, 3) carrying out light expose on ice to prevent thermal damage

(Burkert et al., 2019) Changes in the Active, Dead, and Dormant Microbial Community Structure across a Pleistocene Permafrost Chronosequence

ABSTRACT SUMMARY

- Microorganisms survive in permafrost for millenia, despite extreme conditions

- Study of permafrost microbial communities relies on DNA-based methods such as metagenomics and 16S rRNA gene sequencing

- DNA-based methods fail to distinguish between active, dead and dormant cells

- Distinguishing between active, dead and dormant cells is particularly important in permafrost where subzero temperatures preserve DNA from dead cells and incidence of dormancy may be high

- To address these issues Burket et al., use three approaches, 1) LIVE/DEAD differential staining couples with microscopy, 2) endospore enrichment, 3) selective depletion of DNA from dead cells

- Cell counts and analysis of 16S rRNA genes from active, dead and dormant cells found that cell capable of forming endospores were not necessarily dormant

- Removal of dead DNA did not significant affect microbial community composition

METHOD

- LIVE/DEAD staining: Use a LIVE/DEAD BacLight bacterial viability kit to differentially stain live and dead cells.

- LIVE/DEAD staining: Add 3 L of a 1:1 mixture of 3.34mM SYTO 9 and 20 mM propidium iodide solution to 1-ml cell suspensions.

- LIVE/DEAD staining: Incubate stained suspensions at room temperature for 15 minutes in the dark to allow dye to permeate the cells and bind to DNA.

- DAPI staining: After removing soil debris, add 3 L of 14.3 mM DAPI stock solution to each 1ml of cell suspension.

- DAPI staining: Incubate stained suspensions at room temperature for 15 minutes in the dark.

- Cell enumeration: Dilute and vacuum filter the stained suspensions onto a 25-mm-diameter 0.2-m-pore-size black polycarbonate membrane. Place on a slide with sterile forceps.

- Cell enumeration: Observed samples at x1,000 total magnification on a single focal plane using a fluorescence microscope with appropriate filters for each stain.

- Cell enumeration: Count 15 fields of view for LIVE.DEAD and DAPI-stained cells for each sample.

- Cell enumeration: Multiple the average number of cells per field of view by the area of the filter. Correct the dilution factor for dry weight and calculate the average number of cells per gram (dry weight).

- Separate cells from the soil matrix for PMA and endospore enrichment – this results in little biomass pellets!

- Depletion of DNA from dead cells via PMA: Resuspend extracted cell pellets in 500 l of 0.85% NaCl (sodium chloride) solution and place in clear 1.5-ml centrifuge tubes.

- Depletion of DNA from dead cells via PMA: Add 2.5 l of 20 mM PMA solution to each microcentrifuge tube, to a final concentration of 100 M (this is increased from the commonly used concentration of 50 M due to the presence of leftover soil debris following cell extraction).

- Depletion of DNA from dead cells via PMA: Incubate the tubes in the dark at room temperature for 10 mins.

- Depletion of DNA from dead cells via PMA: Place the tubes on a sheet of foil in an ice bucket to prevent warming. Place a 500-W halogen lamp 20cm above the samples for 15 mins, mixing the samples gently every 5 minutes to ensure even light distribution.

- Depletion of DNA from dead cells via PMA: Centrifuge the samples at 10,000 X g for 15 minutes and discard supernatant.

- Depletion of DNA from dead cells via PMA: Store the PMA treated pellets at -20C until downstream DNA extraction.

- Depletion of DNA from dead cells via PMA: An additional method verification step for soil was used.

- Finally endospore enrichment was carried out.

I will be using the PMA treatment with three substrates: snow (spring), ice (spring and summer) and cryoconite (summer). exDNA from melted samples (snow/ice) will likely pass through the filter. The results of my PMA treatment with snow and ice will let me quantify DNA from intact cells vs DNA from compromised cells (not inc. exDNA).

My Activities:

- Read and write out in my own words the methods from both papers

- Using the methods from both papers plan ice and snow incubations (What sample volumes do I need/will I be using? What PMA concentrations do I need? I will likely be filtering 100-400ml of snow and ice onto a filter per sample. PMA incubations can be done on melted samples in a whirlpak bag before filtering.)

- Using the methods from both papers plan cryoconite incubations

- Write these plans into my personal supraglacial PMA protocol

BASIC PMA PROTOCOL

- Put samples in 1.5 ml centrifuge tubes.

- Add PMA to half of the sample tubes.

- Add phosphate buffer to the non-PMA-treated sample tubes.

- Incubate all tubes in the dark at room temperature/25C for 15-60 minutes (possibly turning by hand every 5 minutes).

- Place all tubes on a sheet of foil on top of ice. At 15-20cm distance, expose to 500 W halogen lamp for 15 minutes, inverting by hand every 5 minutes.

- (Centrifuge samples at 10,000 X g for 15 minutes and discard supernatant).

- Store at -20C - -80C until DNA extraction.

ICE PMA PROTOCOL

- Melt ice samples at room temperature.

- Place 400 mL of ice melt into a 1 L Whirlpak bag.

- Add 2 mL (2000L - microlitres) of 20mM (millimolar) stock solution of PMA to the 400 mL samples to a final concentration of 100 M (micromole per litre or micromolar) (Malard et al., 2019).

﻿5 μl PMA to 2ml of liquid = final PMA concentration of 50 m

10ul PMA to 2ml liquid = 100um

2000ul PMA t0 400ml liquid = 100um

SNOW PMA PROTOCOL

CRYOCONITE PMA PROTOCOL

References

Burkert, A., Douglas, T. A., Waldrop, M. P., and Mackelprang, R. (2019). Changes in the Active, Dead, and Dormant Microbial Community Structure across a Pleistocene Permafrost Chronosequence. Appl. Environ. Microbiol. 85. doi:10.1029/2008GB003327.

Malard, L. A., Šabacká, M., Magiopoulos, I., Mowlem, M., Hodson, A., Tranter, M., et al. (2019). Spatial variability of Antarctic surface snow bacterial communities. Front. Microbiol. 10, 1–12. doi:10.3389/fmicb.2019.00461.

Ramírez, G. A., Jørgensen, S. L., Zhao, R., and D’Hondt, S. (2018). Minimal Influence of Extracellular DNA on Molecular Surveys of Marine Sedimentary Communities. Front. Microbiol. 9, 1–12. doi:10.3389/fmicb.2018.02969.