From ﻿Interpreting spatial patterns in redox and coupled water–nitrogen fluxes in the streambed of a gaining river reach (Heppell et al., 2014)

Table 1. Techniques available through QMUL School of Geography labs. LOD = limits of detection. IC = ion chromatography. ﻿ICP-OES = inductively coupled plasma optical emission spectrometry.

| Chemical analysis | Method | Machine | LOD |
| --- | --- | --- | --- |
| Chloride | IC | ﻿Dionex-ICS2500 | ﻿0.7 mg L-1 |
| Sulphate | IC | ﻿Dionex-ICS2500 | ﻿0.3 mg S L-1 |
| Nitrate | IC | ﻿Dionex-ICS2500 | ﻿0.04 mg N L-1 |
| Ammonium | Colorimetric analysis using adapted indophenol blue method | ﻿Seal AQ2 | ﻿0.03mgN L-1 |
| Fe | ﻿ICP-OES | ﻿Varian Vista-Pro | ﻿0.01 mgL-1 |
| Fe(II) | ICP-OES | Varian Vista-Pro | ﻿0.04 mg L-1 |
| Dissolved Organic Carbon (DOC) | Thermal oxidation using the non-purgeable organic carbon method | ﻿Thermalox TOC/TN Analyzer | ﻿1.0 mg L-1 |
| Oxygen | Oxygen electrode connected to an in-line amplifier, connected to a data-logging meter | Oxygen electrode | ﻿0.1 mg L-1 |

Table 2. Sampling strategies used by Heppell et al., (2014) for physio-chemical analysis of pore water.

| Chemical analysis |  |
| --- | --- |
| Initial Collection | Flush syringe and plastic tubing with pore water. Extract water samples using a syringe and plastic tubing |
| NO3-, NH4+, SO42-, Cl- (anions and cations) | Filter through 0.45 m cellulose acetate membrane in the field and store in prewashed polycarbonate bottles. |
| Fe, Mn | Filter through 0.45 m cellulose acetate membrane in the field and store in prewashed polyethylene tubes and acidified with HNO3 in the field. |
| After filtering | Store at 5C on ice until further analysis . |
| Additionally | Collect blanks for each analyte. |
| Dissolved organic carbon (DOC) | Filter through 0.45 m cellulose acetate membrane and store in acid-washed amber glad bottles and acidifie to pH <2 with HCl in the field |
| Fe (II) | Buffered 1,10-phenathroline method: 1mL pore water filtered through oxygen free nitrogen-flushed 0.45 m filter into 4mL preservative solution. Stored in dark until UV-spectrophotometer. |
| Oxygen | Calibrated, fast response oxygen electrode connected to an in-line amplifier, connected to a data-logging meter. LOD = ﻿0.1 mg L-1 |

Tasks

- Write down indicative concentration ranges that I might find in my samples for each analyte.

- Find out how sensitive the methods already identified are. Do they meet the levels we think we’ll find? If so, great. Let’s organise using them. If not, what other methods might be available?

- Do I need to filter out separate samples for anion, cation and DOC analysis? Or one for cation/anion and one for DOC? What volume do these samples need to be?

- Review literature for glacial DOC methods.

- Speak to Michelle Day about cation acidification protocols

- What are usual DOC concentrations for glacier surfaces?

Additional notes:

- IC will not detect very low levels of phosphate so automated colorimetric flow injection analysis method should be used.

- Need to bring plastic tubes for dissolved ions analysis

- Bradley: previous work with DOC needed samples to be filtered using glass syringes and rinsing with acid

- Bradley: DOC problematic. Plastic syringes not good for glacial samples. GFZ group uses glass syringes and metal housing for filter papers. The filters are ashed GFF filters. If metal filter housing and glass syringes used then also need HCl to clean them in the field. This is acid! Dangerous! Indeed samples can be stored cold in the amber vials after they have been passed through a 0.2 or 0.7um filter, without the need for acidification in the field, so long as they will be returned to lab and processed fairly soon. The acidification can then be done in the lab.

- Bradley: Can’t melt snow/ice in plastic whirlpak bags and leave to melt. Will need to bring many large glass jars. I would need to prep and bring these.

- Jungblut: To measure DOC, colleagues in Canada filter water samples through prerinsed cellulose acetate filters (0.2m) (also glass fibre filter)?. Samples are then stored in acid washed and combusted glass vials at 4C in the dark until analysis with Simadzu TOC-V analyser. TOC is measured using TOC-L analyser with unfiltered water.

- Heppell: Filters for cation, anion and DOC analysis of aqueous samples have previously used 0.2m pore size polypropylene filters in polypropylene housing, 13mm diameter = 10ml sample (https://www.sigmaaldrich.com/catalog/product/aldrich/wha67851302?lang=en&region=GB)

- Heppell: Has also previously used 0.2m cellulose acetate filter papers for cation and anion analysis, but wouldn’t recommend them for DOC analysis. 0.7 um GF/F seems to be the usual filters for DOC from glacial literature.

- Vials for anion and cation analysis: We have used the following 15 mL and 50 mL tubes in the past (exact brand may vary) (https://www.coleparmer.co.uk/i/cole-parmer-conical-bottom-centrifuge-tube-pp-15-ml-plastic-rack-300-cs/0633689?PubID=UY&persist=true&ip=no&keyword=&gclid=Cj0KCQiA-aGCBhCwARIsAHDl5x-r3pzqWzuVmQyQknvfNCjzT1l5hqfA3PjPg2n4HfGyuHstqBF0KPEaAlYGEALw_wcB&gclsrc=aw.ds)

- Vials for DOC analysis: amber glass, combusted. You would want to cover caps with pre-combusted foil (https://www.coleparmer.co.uk/i/thermo-scientific-b7800-3a-glass-vials-ptfe-lined-cap-amber-8-ml-200-pk/0892233?PubID=UY&persist=true&ip=no&keyword=&gclid=Cj0KCQiA-aGCBhCwARIsAHDl5x-ozUpi2R7ZqEMY_x_IrsiNiV2Bk8V6YjOa9VGWtUTHVR9xXjTF-kMaAk8_EALw_wcB&gclsrc=aw.ds)

- Cation and DOC vials need to be acidified: Kate has previously carried this out in the field for DOC with HCl (35µl 33% HCl PER 5 mL SAMPLE) but is this common for glacial studies? Acidification of cation samples can be carried out back in lab in UK. Acidification keeps the analytes of interest in the solution prior to being nebulized. Does this need to be done for anion samples?

Syringe Filters (https://scientificfilters.com/syringe-filters-aqueous-solutions/#:~:text=Polypropylene%20(PP)%20%E2%80%93%20can%20be,and%20aromatic%20or%20chlorinated%20solvents.)

- Types of filters: nylon, polyethersulfone, cellulose acetate

- Their hydrophilic properties mean they filter water efficiently

- Hydrophobic syringe filters (i.e. those made of stuff like Teflon and speciality plastics) can also be used by wetting the membrane with alcohol

- When choosing syringe filters consider how they are being used: think about filter connection, filter material, pore size, diameter of the filter and sterility.

- Connection type: generally have female luer lock inlets and male luer slip outlets. However syringe filters can also be fitted with what is called a double luer lock connection which is a female luer lock inlet and a male luer lock outlet.

- Membrane material, things to consider: chemical compatibility (e.g. pH), flow rate, binding properties (will some of the stuff I’m investigating bind to the filter?), extractables (do bits of filter end up in my sample?), wettability of the membrane (hydrophilic or hydrophobic)

- Common membrane materials: polyethersulfone (PES), nylon, cellulose acetate, regenerated cellulose, anopore, glass fibre, polypropylene

- Pore sizes: 0.05 µm, 0.10 µm, 0.22 µm, 0.45 µm, 0.70 µm, 0.80 µm, 1 µm, 3 µm, 5 µm, and 10 µm

- Diameter: Syringe filter size is determined by the size of your sample. In other words, the amount of volume you plan to push through the filter. As a general rule, use a syringe filter with a greater membrane area if you plan to push a high volume of aqueous solution through it.

- Syringe filters for aqueous solutions are available in diameters of: 4mm, 13mm, 17mm, 20mm, 25mm, 30mm, and 33mm.

- Here's a guide to help you choose your syringe filter size:

- 4 mm filters - for small volume samples less than 1 ml

- 13 mm filters - for sample volumes between 1 and 10 ml

- 17 mm filters - for sample volumes greater than 10 ml

- 25 mm filters - for most applications, ideal for high throughput labs

- 30 mm filters - for large sample volumes greater than 100 ml

- Sterile if you want a sterilized aqueous solution, nonsterile if you’ll be sterilizing later

DOC Questions:

- Are there enough glass jars in the lab to melt my samples for DOC?

- Can I get these jars prepped and shipped to Svalbard in time, or alternatively, can I bring them in my luggage?

- Is it appropriate to use plastic syringes for DOC samples?

- What other methods are community used for supraglacial DOC analysis, apart from using glass syringes/metal housing/glass fibre filters/HCl?

- If plastic syringes are not appropriate and there are no other alternatives, can I acquire glass syringes/metal housing/glass fibre filters/HCl and get the training to use them safely in the field?

Cation/Anion Questions:

- Can I use plastic syringes for cation/anion samples?

- What volume will my cation/anion samples be?

- What filers do I need for cation/anion samples?

- Do I need to filter separate samples for cation/anion analysis?

- How can I get the right syringes/filters for my cation/anion samples? What do I need to do to prepare this equipment?

- Can I use ion chromatography (IC) at QMUL for anion analysis?

- Should I use automated colorimetric flow injection analysis for measuring phosphate instead of IC, due to IC limits of detection?

- Can I use inductively coupled plasma mass spectrometry at QMUL for cation analysis?

- Do I have time to do this myself, or is it more practical to have this done elsewhere?

Microbial Questions:

- Do I have all the equipment I need for collecting microbial samples?

Other Analyses Questions:

- Is there a pH meter I can use that is sensitive enough?

- Is there a conductivity meter that is sensitive enough?

- Is there a GPS/elevation device I can use to record the location of my samples?