DOC, and dissolved nutrient method (Holland et al., 2019)

Sample Collection

- Furnace glass jars at 500C prior to use

- Ice for nutrient analysis placed in Whirl-Pak bags (+ procedural blanks).

- Ice for DOC transferred to glass jars that had been rinsed 3 times with sample (+ procedural blanks).

- Leave ice samples to melt overnight.

- Ice melt samples for nutrient analysis filtered through 25mm, 0.22 m cellulose nitrate inline syringe filter (Whatman) and stored in high-density polyethylene plastic bottles (Nalgene 30mL). Freeze bottles and store at -20C. (+ procedural blanks).

- Wash glass filtration column three times with sample water prior to collection of the filtrate. (+ procedural blanks).

- Ice melt samples for DOC analysis filtered using glass filtration column and a furnaced 47mm, 0.7 m GF/F. (+ procedural blanks).

- Store filtrate in pre-furnaced amber glass vials and acidify with 100 L of 1M HCl. Chill samples to 3C and maintain temperature during transport. (+ procedural blanks).

- Thaw nutrient samples immediately prior to analysis using a 40C hot water bath. (+ procedural blanks).

Sample Analysis

- Quantification of DIN species (NH4+, NO2-, NO3-): NH4+ quantified spectrophotometrically using flow injection analyser. Measurements based on phenolate-hypochlorite alkaline reaction method measured at 630 nm (LOD = 0.62 M).

- Quantification of DIN species (NH4+, NO2-, NO3-): Quantify NO2- and total oxidized nitrogen (TON, NO2- and NO3-) spectrophotometrically using an automated photometric analyser. (Calculate NO3- but subtracting NO2- from TON).

- Purify potassium persulfate by recrystallization to remove N contamination.

- Quantify total dissolved nitrogen (TDN) by digesting samples with purified potassium persulfate, sodium hydroxide and boric acid reagent and autoclaving at 121C for 30 mins. Measurements of TDN based on hydrazine-sulphanilamide reaction method measured at 540 nm. (DON = TDN-DIN). LOD = ﻿0.14µM (NO2-), 0.64µM (TON) and 0.87µM (TDN/DON).

- Quantify total dissolved phosphorus (TDP) by digesting samples with purified potassium persulfate, sodium hydroxide and boric acid reagent and autoclaving at 121C for 30 mins. Measurements of TDP based on hydrazine-sulphanilamide reaction method measured at 540 nm. (DOP = TDP=DIP).

- Quantification of DIP (PO43-): Quantify PO43- in digested and undigested samples with a flow injection analyser using the molybdenum blue methods measured at 880 nm. DOP= TDP (digested sample) – DIP (undigested sample). LOD = 0.02 µM.

- Quantification of DOC: Use a Shimadzu TOC-L Organic Carbon Analyzer with a high-sensitivity catalyst.

- Measure non-purgeable organic carbon (NPOC) after acidification of samples with HCl and catalytic combustion (680C) of dissolved organic carbon to carbon dioxide, then measure by infrared absorption. LOD=9.5 µM.